# Anatomy of a block

A block is a single or multiple pulley.

A block contains one or more sheaves held between cheeks or chocks.

A block is often fixed to the end of a line, to a spar, or to a surface.

A line (rope) goes around the sheaves, and maybe through one or more matching blocks to make up a tackle.

By adding more blocks to the tackle you can improve mechanical advantage (make lifting easier).

A block can be used to improve mechanical advantage or to change the direction of the line.

A block can include a ratchet that allows the block to turn freely in one direction but not in the other direction making the load easier to hold.

A block can be made from different material.  A wooden block is measured by size - length from crown to tail measured in millimetres around the shell; and it will take a rope one-tenth of its size. (Rope measured by its diameter in millimetres).   Metal Blocks are measured by the size of rope for which they are designed for and which is usually marked on a cheek plate.

Blocks can be internal bound where the sheave is encased by a fixed wood and / or metal frame (as above), others can be snatch blocks (left) that have an opening gate to allow the rope to be placed in the block easily.  A snatch block is not at safe as a fixed internal bound block as the gate could open and should not be used where lives rest on them and where an internal bound block is available.

A snatch block requires more maintenance, including oil / grease – avoid too much oil/grease as it can collect dirt/grit and cause excessive wear.

# Types of block

A block is referred to an internal bound is where the sheaves are encased in the block and the rope has to be passed through the swallow and over the sheaves, or a snatch block if it has a side opening gate allowing the rope to be placed in from the side.

A block could include a ratchet (ratchet block) that allows the sheave to turn in one direction but not the other – a retched block can sometimes have a mechanical switch to turn on/off the ratchet.

A wooden block is measured by its size (millimetres) from crown to tail and the rope diameter should be 1/10 of that length, and others based on the diameter of rope that they should be used for.

A block will have a connector, this will often be an eye at one end, but this could be replaced by a hook.  A block could have a hook or eye at the other (tail) end (a becket).  The eye or hook may swivel or rotate.  Depending how the eye or hook moves depends on the name given to it.

A shackle, also known as a gyve, is a U-shaped piece of metal secured with a clevis pin or bolt across the opening, or a hinged metal loop secured with a quick-release locking pin mechanism.  The pin in the shackle can have a locking mechanism which could include a nut, wire, pin, string.

# Mechanical Advantage

To lift 100kg weight would take 100kg of effort, however if you can add mechanical advantage you can change that.  Mechanical advantage is added by adding blocks in a correct order.  When blocks are added together they become tackle.  The tackle will have a hauling part (bit you pull) and a standing part fixed at the opposite end the hauling part either to the becket of a block or to the object to be lifted.

The rope that passes through two or more blocks is called a rove and you are said to rove to advantage when the hauling end is attached to the standing block and rove to disadvantage when the standing end so fastened to the moving block.  This is because fastening the hauling end to the standing block adds more mechanical advantage.

When lifting an item with a single fixed pulley the effort needed to lift the item will be equal to the load (weight) of the item if we ignore friction – in reality we will need a bit more effort than the load because of fiction.

If we add more blocks, then we can reduce the effort needed to move the load.  However, we will also need to pull the rove more.  In the simple one block image (right) to lift a 100kg load 1m we would need 100kg of effort and we would need to pull the rove 1m.

If we add a second block as per the left image we would now only need 50kg of effort to move the 100kg load but we would also need to pull the rope 2m to get the load to move 1m.  This is achieved by each of the falls carrying equal load each – 50% of the load.

As we add blocks and falls we can increase the mechanical advantage.  If we add another block so that we have three falls (right) we now have a 3: 1 mechanical advantage.  In the load is 100kg each fall is carrying 1/3 of the load = 33.3kg each.  To move the load by 1m we would need 33.3kg of effort and would need to move the rope 3m.

Depending if the block is fixed or moving and how many falls there are the tackle is given different names (right).

When the load has been lifted it needs to be held in place before being lowered and this can be done by chocking or racking.

# Holding the load

Once a load has been lifted using blocks it will need to be held until lowered and this is achieved by chocking the luff or by racking.

Racking is a temporary holding solution.  One method is simply to hold the running parts together (if not all of them then the opposite ones), or the running and standing parts together with the hands.   If the load is too heavy then add a racking turn by passing a short length of line, taking figure of eight turns around the standing and running parts, the end being held in the hand or secured around the parts. The hauling part is kept clear of the racking in order for it to be moved.

The alternate method is choking the luff which again is a temporary method and should only be used on light loads as it can damage the rope (rove).  A bight (bend in the rope) of the hauling part is pulled through the centre of the moving parts and wedged between the moving parts and the block.  Choking can come undone if the weight of the moving part is heavy and you drop the rope is can pull the bight free and the load drop uncontrollably.

# Hooks

There are different types of hooks with the hook being open or closed.  An open hook, yellow, is a simple hook but if used incorrectly the load could slip off the hook.  A closed hook (red) has a gate across the hook that opens one way or has a locking mechanism to stop the load falling off accidentally.

Correct loading of a hook is critical – it should sit level with the ground and not tipped forwards, sideways or backwards.

The point of the hook is called the bill, the body is called the shank, and the bottom the crown; the part opposite the bill is the back; the jaw is the space between the bill and the top of the shank, and the clear is the inside diameter of the crown.

If a hook does not have a gate it can be moused to stop the load falling off the hook accidentally.  Alternatively, a shackle can be moused to stop the pin falling out or a gate on gate hook can be moused to stop it coming undone accidentally.

Mousing is a length of line or wire.

# Shackles

Shackles are fastening used for joining ropes, webbing and chain together and are usually made from metal, especially from carbon magnesium steel.

If a Shackle is supplied from HM Dockyards they often show a test load and a test date most commercial shackles do not.

Shackles close in one of three ways

- Screw shackle: The end of the bolt passes through one lug and is screwed into the other lug.  The bolt has a head and of under load should be moused.

  - Wire mouse: a length of wire is used and three or four turns are passed around the lug and through the eye of the pin and the ends are then twisted tightly together. Surplus wire is cut off.

- Fore-lock shackle: The end of the bolt projects beyond one of the lugs and a flat tapered split-pin (fore-lock) is passed through a slot in the end of the bolt. It is secured by opening the jaw of the pin. The fore-lock may by attached to the shackle by a keep chain or a length of wire.

- Split pinned shackle: This type of shackle is of similar design to the fin and fore-lock shackle, but is supplied with a galvanised split pin to serve the same purpose as a fore-lock. The pin may be attached to the shackle by a keep chain or a length of wire.

The parts of a shackle are:

# Thimbles

Thimbles are spliced into the end of a fibre or wire rope to take the chafe of a shackle or shackle bolt and also to support the eye formed in the rope.

Thimbles are classified according to the diameter of the rope for which they are intended and also their shape; most thimbles being manufactured of galvanised steel, although they may be made of stainless steel, phosphor bronze or polyamide.

Thimbles are either solid, round or heart-shaped and open or welded at the neck; the gap formed at the neck can be sprung open to allow the eye of a tackle hook or lug of a shackle to enter.